The Mustang is going into hibernation for 7 months.

Have a 10X20 Coverall with a Gravel Floor, 10 in x 16 Ft planks to put the Car on, AC power and a 2-Watt Solar panel.

This is what I did for Storage;

Clean and Wax, Buff and especially no Watermarks.

Remove all MP3 discs and turn off Shaker 500.

Full Tank of Gas mixed with Stabilizer.

Inflate Tires to 35 lbs..

Oil changed.

Block Heater Cord out but not plugged in.

Solar panel attached to Battery.

Air Filter inlet Blocked with Pot Scrubbers and a few MothBalls in Filter Box.

Sonic Rodent Repeller plugged in under vehicle.

Warferin Sacks in each corner of the Coverall.

Moth Balls under the Car .

Crack the windows slightly, so not pressing Seal.

Crack the Top so it is just loose but fabric is Tight.

Place Drier Fabric Softeners in trunk, and in Passenger Compartment, The smellier the better.

Car Cover on (removed Antenna).

Check Battery every Month, Solar panel will keep it at 12.8 volts with Computer and alarms working.

Do Not Start the car until your ready to take it out of Storage. Starting with cold Oil drained to the bottom of the Engine is BAD.

Results from last Year, Car was ready to go, Computer still running and when I cranked the engine it fired and ran in about 2 seconds. Went for a slow ride about Town just to Check. Then drove 15 Miles at speed to the Dealer to Change Oil and a 40 point Check.  Battery showed as questionable but tested good on my last Oil Change. Deflate Tires to 32 lbs.